Table for CLiq parameters

| Parameters | Value/type | Reason |
| --- | --- | --- |
| Calculation method | Robertson and Wride (NCEER 1998, 2009) | This is prevalent within the literature as a common calculation method for liquefaction potential. |
| Peak Ground Acceleration (PGA) | 0.2- 0.3 g | Based on NZ Transport Agency’s Bridge manual- same value for shallow soil site and deep or soft soils which I think fits Hinuera? |
| Earthquake magnitude (Mw) | 6- 7 Mw | Based on evidence from Kerepehi Fault research |
| Ground water level (m) | 2-4 m | Based on opus report for Waikato Expressway. |
| Fines content | Yes | Included in analysis as when removed it tends to underestimate the degree of liquefaction. |

Water info from: Opus (2014). Ruakura Interchange, Assessment of Water Effects. Waikato Expressway: Hamilton Section: 10.

https://www.waikatoregion.govt.nz/environment/natural-resources/water/groundwater/groundwater-levels/hamilton-basin/

Peak ground acceleration from: Agency, N. T. (2014). The NZ Transport Agency’s Bridge manual. Site Stability, Foundations, Earthworks and Retaining Walls: 6-7.

https://www.eqc.govt.nz/sites/public_files/3609-Prob-seismic-hazard-NZ%282of2%29_2.pdf

http://www.tandfonline.com/doi/pdf/10.1080/00288306.1998.9514816

Magnitude from: Persaud, M., Villamor, P., Berryman, K. R., Ries, W., Cousins, J., Litchfield, N., & Alloway, B. V. (2016). The Kerepehi Fault, Hauraki Rift, North Island, New Zealand: active fault characterisation and hazard. New Zealand Journal of Geology and Geophysics, 59(1), 117-135.

https://www.boprc.govt.nz/media/160681/earthquake_hazards_of_the_bay_of_plenty_region_-_update_to_1999.pdf

Persaud, M., Villamor, P., Berryman, K. R., Ries, W., Cousins, J., Litchfield, N., & Alloway, B. V. (2016). The Kerepehi Fault, Hauraki Rift, North Island, New Zealand: active fault characterisation and hazard. New Zealand Journal of Geology and Geophysics, 59(1), 117-135.

This simple calculation suggests recurrence intervals on individual segments of 4.0–6. 0 ka are common on the central segments but may be less frequent on the Okoroire, Elstow and Te Puninga segments where total throw on the main strands of the segments is smaller.