Table for CLiqTM parameters

| Parameters | Value/type | Reason |
| --- | --- | --- |
| Calculation method | Robertson and Wride (NCEER 1998, 2009) | This is prevalent within the literature as a common calculation method for liquefaction potential. |
| Peak Ground Acceleration (PGA) | 0.38 g | Based on NZ Transport Agency’s Bridge manual, Geotechnical earthquake engineering practise and NZS1170.5:2004 Structural design standard. |
| Earthquake magnitude (Mw) | 6- 7 Mw | Based on evidence from Kerepehi Fault research |
| Ground water level (m) | 2-4 m We will put 1 m | Based on opus report for Waikato Expressway. |
| Fines content | Yes | Included in analysis as when removed it tends to underestimate the degree of liquefaction. |

Water info from: Opus (2014). Ruakura Interchange, Assessment of Water Effects. Waikato Expressway: Hamilton Section: 10.

When looking at the geotechnical database where the actual data was derived from and there were instances where the water level was in countered at 0.6 and 0.9 depths so under a meter.

Magnitude from: Persaud, M., Villamor, P., Berryman, K. R., Ries, W., Cousins, J., Litchfield, N., & Alloway, B. V. (2016). The Kerepehi Fault, Hauraki Rift, North Island, New Zealand: active fault characterisation and hazard. New Zealand Journal of Geology and Geophysics, 59(1), 117-135.

https://www.boprc.govt.nz/media/160681/earthquake_hazards_of_the_bay_of_plenty_region_-_update_to_1999.pdf

Persaud, M., Villamor, P., Berryman, K. R., Ries, W., Cousins, J., Litchfield, N., & Alloway, B. V. (2016). The Kerepehi Fault, Hauraki Rift, North Island, New Zealand: active fault characterisation and hazard. New Zealand Journal of Geology and Geophysics, 59(1), 117-135.

Peak ground acceleration (PGA)

Equation from: Ministry of Business Innovation and Employment, & New Zealand Geotechnical Society. (2016). Earthquake Geotechnical Engineering Practice.

amax=co,1000r1.3fg

where

- Co,1000 is the Unweighted peak ground acceleration coefficient corresponding to a 1000 year return period (in our case it is classified on the bases of the field area consisting of shallow soil = 0.2 (from Ministry of Business Innovation and Employment, & New Zealand Geotechnical Society. (2016). Earthquake Geotechnical Engineering Practice.

- r is the return period factor= 1.8 1/2,500 yrs (from Standards New Zealand. (2004). Structural Design Actions)

- f is the site response factor= 1.33 for shallow soils (class C). (from Ministry of Business Innovation and Employment, & New Zealand Geotechnical Society. (2016). Earthquake Geotechnical Engineering Practice.) For shallow soils, class C. Decision was based on data derived from the geotechnical database, the same data that was used for this research. 7 were looked at and all had values over 25 kPa at 20 m depths.

Therefore

PGA=0.21.81.31.33

=0.37g

Can also be calculated as:

From New Zealand Geotechnical Society. (2010). Geotechnical earthquake engineering practice.

And

NZS1170.5:2004 Structural design standard.

ah=ZRC

Z- Hazard factor= 0.16

R- Return period factor= 1.8 for 1/2500

C- Site response factor= 1.33 for shallow soils (class C) and 1.12 for deep soft soils (class D).

Therefore:

ah=0.16*1.8*1.33

=0.38g

| Parameters- equation 1- | SLS (1/25 yrs) | ULS (1/500 yrs) | MLS (1/2,500 yrs) |
| --- | --- | --- | --- |
| Co,1000 | 0.2 | 0.2 | 0.2 |
| ru | 0.25 | 1.0 | 1.8 |
| f | 1.33 | 1.33 | 1.33 |
| PGA=C0,1000 x ru1.3x f | 0.05 g | 0.20 g | 0.37 g |

| Parameters- equation 2- ah= | SLS (1/25 yrs) | ULS (1/500 yrs) | MLS (1/2,500 yrs) |
| --- | --- | --- | --- |
| Z | 0.16 | 0.16 | 0.16 |
| R | 0.25 | 1.0 | 1.8 |
| C | 1.33 | 1.33 | 1.33 |
| ah=Z R C | 0.05 g | 0.29 g | 0.38 g |