Appendices xx: Soil Profile Descriptions

Profile 2

| Geological name | Depth (cm) | Layer description |
| --- | --- | --- |
| Fill | 0-15 | FILL |
| Hamilton Ash Fmn | 15-26 | clayey SILT; dark-brown (10YR 3/4); moist; apedal earthy; weak, friable; slightly sticky; slightly plastic; diffuse, irregular lower boundary. |
| Hamilton Ash Fmn | 26-32 | Silty CLAY; yellowish brown (10YR 5/6), few fine dark brown mottles (10YR 3/4); moist; low pedality, few fine polyhedral peds; slightly firm, brittle; moderately sticky; moderately plastic; no roots; light grey silty infilled cracks ~ 1cm aperture; diffuse, irregular boundary. |
| Hamilton Ash Fmn | 32-89 | Silty CLAY; bright brown (7.5YR 5/6); moist; moderate pedality, few fine nutty peds; firm, brittle; moderately sticky; moderately plastic; no roots; diffuse, irregular lower boundary. |
| Hamilton Ash Fmn | 89-117 | Silty CLAY; brown (10YR 4/6); moist; moderate pedality, common fine to very-fine nutty peds; slightly firm, brittle; moderately sticky; moderately plastic; no roots; diffuse, irregular lower boundary. |
| Hamilton Ash Fmn | 117-226 | Silty CLAY; variable colours/profuse mottles. Dull yellow-orange (10YR 7/3), dull orange (7.5YR 7/3), yellow-orange (7.5 YR 7/8), orange (7/5YR 4/6); moist; apedal earthy; slightly firm, brittle; moderately sticky; very plastic; common fine to very fine rootlets at 640 cm, few coarse roots along discontinuities; brown (7.5YR 7/8) in cracks ~1 mm aperture, light grey (7.5YR 8/1) in veins ~ 1 cm diameter; diffuse, irregular lower boundary. |
| Rangitawa Tephra (H1) | 226-263 | Clayey SILT, fine sand (mica) at top; light brownish grey (7.5YR 7/2); brownish black (7.5YR 3/2) and bright brown-orange (7.5YR 5/8 – 6/8) in cracks, light grey (10YR 8/1) along veins; slightly moist; apedal earthy; platy breakage; hard, brittle; slightly sticky, moderately plastic, few fine rootlets (not alive); sharp, irregular lower boundary. |
| Rangitawa Tephra (base layer) | 263-266 | Silty SAND (fine to medium), some clay; light orangey-yellow (10YR 8/3); slightly moist; apedal, single grained; non-sticky; non-plastic; no roots; (mica, quartz, manganese…) abrupt, irregular lower boundary. |
| Kauroa Ash Fmn | 266-294 | Silty CLAY, some fine sand; dark reddish brown (5YR 3/6); moist; apedal earthy, blocky breakage; slightly firm, semi-deformable; moderately sticky; very plastic; many extremely fine to microfine roots; gradational lower boundary. |
| Kauroa Ash Fmn | 294-380 | Clayey SILT; bright brown (7.5YR 5/8); moist; apedal earthy; slightly firm, semi-deformable; slightly sticky, moderately plastic; common extremely fine roots; diffuse, irregular lower boundary. |
| Walton Subgroup | 380-834 | Gravelly CLAY/SILTS; variable colours; white, pale-yellow (5YR 8/4), yellow (5YR 8/6), light yellow-orange (7.5YR 8/3), orange (7.5YR 7/6), orange (7.5YR 6/8 and 10YR 7/8); moist; apedal; slightly firm to firm, deformable; slightly sticky; slightly plastic; some fine roots along discontinuities; poorly sorted, poorly graded; abrupt, wavy boundary. |
| Walton Subgroup | 834-914 | Silty CLAY; white with interbedded layers (of pinks/reds etc); wet; apedal earthy; weak, deformable; slightly sticky; moderately plastic; some fine roots along discontinuities; poorly sorted, poorly graded; boundary not observed (End of profile) |

Profile 3

| Geological name | Depth (cm) | Layer description |
| --- | --- | --- |
| Fill | 0-83 | FILL |
| Hamilton Ash Fmn | 83-119 | Clayey SILT; dark-brown (10YR 3/4); slightly moist; moderate pedality, few fine polyhedral peds; weak, friable; slightly sticky; moderately plastic; common fine roots in upper portion; irregular, diffuse lower boundary. |
| Hamilton Ash Fmn | 119-161 | Silty CLAY; dull yellowish-brown (10YR 5/4); few fine dark-brown mottles (10YR 3/4); moist; strong pedality, abundant fine polyhedral peds; weak, friable; slightly sticky; slightly plastic; no roots; irregular diffuse lower boundary. |
| Hamilton Ash Fmn | 161-235 | Silty CLAY; bright brown (7.5YR 5/8); red streaking (xxx); moist; weak pedality, common fine to very-fine nutty peds; slightly firm, brittle; slightly sticky; very plastic; abundant microfine rootlets; irregular, diffuse lower boundary. |
| Hamilton Ash Fmn | 235-256 | Clayey SILT; brown (10YR 4/6); brown (7.5YR 4/6) in cracks ~1mm aperture, light grey (7.5YR 8/1) in veins ~1 cm diameter; moist; moderate pedality, few fine polyhedral peds; slightly firm, brittle; slightly sticky; slightly plastic; no roots; infilling at 840 cm (length), some fine mica and micro-fine sub-rounded quartz present; diffuse, irregular lower boundary. |
| Hamilton Ash Fmn | 256-349 | Clayey SILT, some fine sand (mica); yellowish brown (10YR 5/8); some orange staining; moist; moderate pedality; few fine nutty peds; slightly firm, friable; slightly sticky; slightly plastic; no roots; increased concentration of mica; diffuse, irregular lower boundary. |
| Hamilton Ash Fmn | 349-448 | Silty CLAY; yellowish brown (10YR 5/6); weathered colours xx; moist; apedal earthy, slightly firm, brittle; slightly sticky; slightly plastic; no roots; diffuse, irregular boundary. |
| Hamilton Ash Fmn (H1?) | 448-490 | Sandy SILT, some clay; light brownish grey (7.5YR 7/3); Brownish black (7.5 3/2) and bright brown-orange (7.5YR 5/8 and 6/8) in cracks, light grey (10YR 8/1) in veins; moist; apedal earthy, platy breakage; hard, brittle; slightly sticky; slightly plastic; no roots; black inclusions (?) increasing concentration at 1520cm (length); diffuse, irregular lower boundary. |
| Rangitawa Tephra (H1) | 490-498 | Sandy SILT; light grey (7.5YR 8/1); slightly moist; apedal earthy; hard, brittle; slightly sticky; moderately plastic; rootlets; common fine mica and quartz (top- low mica…high mica….low mica-more clay, manganese- base); sharp, irregular lower boundary. |
| Rangitawa Tephra (base layer) | 498-501 | Silty (medium) SAND; light yellow-orange (10YR 8/3) and yellowish orange (10YR 8/8); moist; apedal, single grained; strength; non-sticky; non-plastic; dilatant; no roots; fine mica, quartz, and dark inclusions, some fine, sub-rounded pumice?; abrupt, irregular boundary. |
| Kauroa Ash Fmn | 501-566 | Silty CLAY; dark reddish brown (5YR 3/6); light grey (10YR 8/1) silty infill along discontinuities at top; moist; apedal earthy, blocky breakage; semi-deformable, firm; slightly sticky; very plastic; abundant microfine rootlets; irregular, gradational lower boundary. Black inclusions(mineralogy?) |
| Kauroa Ash Fmn | 556-630 | Clayey SILT; bright brown (7.5YR 5/8); moist; apedal earthy; semi-deformable, firm; slightly sticky; moderately plastic; no roots; diffuse, irregular lower boundary. |
| W? | 630-763 | Silty (fine to coarse) SAND, some clay and fine gravel; variable colours; white, pale-yellow (5YR 8/4), yellow (5YR 8/6), light yellow-orange (7.5YR 8/3), orange (7.5YR 7/6), orange (7.5YR 6/8 and 10YR 7/8); moist; apedal earthy; slightly firm, brittle; slightly sticky; moderately plastic; no roots; poorly sorted, poorly graded; sharp, wavy lower boundary. |
| Walton Subgroup | 763-949 | Silty CLAY; white with interbedded layers (xx, xx); wet; apedal earthy, platy breakage; weak, semi-deformable at top, firm, deformable at base (of profile); slightly sticky; moderately plastic; abundant fine roots along discontinuities; interbedded, poorly sorted, poorly graded; lower boundary not observed (End of profile). |

Profile 4

| Geological name | Depth (cm) | Layer description |
| --- | --- | --- |
| Fill | 0-263 | FILL |
| Hamilton Ash Fmn | 263-283 | Clayey SILT; dark-brown (10YR 3/4); common very-fine red (10R 4/8) mottles; slightly moist; apedal, earthy; slightly firm, friable; slightly sticky; moderately plastic; no obvious roots; irregular, diffuse lower boundary. |
| Hamilton Ash Fmn | 283-318 | Silty CLAY; dull-yellowish brown (10YR 5/3); some fine dark-brown mottles (10YR 3/3) and microfine red-orange veins; moist; strong pedality, abundant fine polyhedral peds; slightly firm, friable; slightly sticky; slightly plastic; no roots; irregular, diffuse lower boundary. |
| Hamilton Ash Fmn | 318-386 | Silty CLAY; bright brown (7.5YR 5/8); abundant fine red mottles; moist; weak pedality, common fine to very-fine nutty peds; slightly firm, friable; slightly sticky; very plastic; no roots; irregular, diffuse lower boundary. |
| Hamilton Ash Fmn | 386-467 | Clayey SILT; yellowish brown (10YR 5/8); common fine red mottles; moist; weak pedality, few fine nutty peds; firm, brittle; slightly sticky; moderately plastic; no roots; diffuse, irregular lower boundary. |
| Hamilton Ash Fmn | 467-553 | Silty CLAY; yellowish brown (10YR 5/6); weathered colours xx; moist; weak pedality, some fine nutty peds, firm, brittle; slightly sticky; moderately plastic; no roots; diffuse, irregular boundary. |
| Rangitawa Tephra (H1) | 553-596 | Fine sandy SILT; light brownish-grey (7.5YR 7/3); very dark brown and reds in cracks, Light grey silty veins; slightly moist; apedal earthy; hard, brittle; slightly sticky; moderately plastic; no roots; common fine mica and quartz (top- low mica…high mica….low mica-more clay, manganese- base); distinct, irregular lower boundary. |
| Rangitawa Tephra (base layer) | 596-599 | Silty (fine to medium) SAND; light yellow-orange (10YR 8/2); moist; apedal, single grained; non-sticky; non-plastic; dilatant; no roots; fine mica, quartz, and dark inclusions (manganese), some fine, sub-rounded pumice; abrupt, irregular boundary. |
| Kauroa Ash Fmn | 599-667 | Silty CLAY; dark reddish brown (5YR 3/6); light grey (7.5YR 8/2) silty infill along discontinuities at top; moist; blocky breakage; semi-deformable, firm; slightly sticky; very plastic; abundant white microfine rootlets; irregular, gradational lower boundary. (mineralogy?) |
| Kauroa Ash Fmn | 667-729 | Clayey SILT; bright brown (7.5YR 5/8); moist; apedal earthy; semi-deformable, slightly firm; slightly sticky; moderately plastic; no roots; very-fine black inclusions; diffuse, irregular lower boundary. |
| Walton Sub-Group Karapiro Fmn | 729-791 | Silty (fine to coarse) SAND, some clay and fine gravel; variable colours; white, pale-yellow (5YR 8/4), yellow (5YR 8/6), light yellow-orange (7.5YR 8/3), orange (7.5YR 7/6), orange (7.5YR 6/8 and 10YR 7/8); moist; apedal earthy; flaky breakage; slightly firm, brittle; slightly sticky; slightly plastic; poorly sorted, poorly graded; sharp, wavy lower boundary. |
| Walton Sub-group Puketoka Fmn | 791-927 | Silty CLAY; white with interbedded layers (---); wet; apedal earthy; flaky breakage along beds; weak, deformable at top, semi-firm, semi-deformable at base (of profile); moderately sticky; moderately plastic; abundant microfine rootlets along discontinuities; interbedded, poorly sorted, poorly graded; lower boundary not observed (End of profile). |